Be it enacted by the People of the State of Maine as follows:

Sec. 1.  12 MRSA §6001, sub-§34-B is enacted to read:

34-B.  Process water.  "Process water" means the water used in the depuration process.

Sec. 2.  12 MRSA §6856, sub-§9 is enacted to read:

9.  Chlorination or ultraviolet radiation.  If a shellfish growing area is closed to the taking, washing and possessing of shellfish due to runoff related to heavy rainfall pursuant to section 6172, subsection 2, a holder of a depuration certificate pursuant to subsection 3 that applies additional water purification treatment to process water through chlorination or ultraviolet radiation is exempt from the shellfish emergency rainfall closures provided the following conditions are met.

A.  If chlorination treatment is used:

(1)  Wash water must contain 0.1 parts per million of residual chlorine for at least one hour of contact time prior to use;

(2)  Treated process water may be used only for wash water; and

(3)  The holder of a depuration certificate pursuant to subsection 3 shall take samples of source water, chlorine-treated water and washed shellfish. Shellfish samples must be analyzed for fecal coliform and water samples must be analyzed for total coliform at a laboratory certified under the National Shellfish Sanitation Program.  The holder of a depuration certificate shall monitor chlorine and salinity levels in the treated process water.

B.  If ultraviolet radiation treatment is used:

(1)  Turbidity may not exceed 20 nephelometric units;

(2)  Turbidity must be measured using a method endorsed by a national organization of public health professionals;

(3)  Process water must contain no detectable coliform bacteria.  The holder of a depuration certificate pursuant to subsection 3 shall continuously treat process water and verify that the disinfection system produces process water with no detectable coliform

organisms as measured using a method approved by the National Shellfish Sanitation Program;

(4)  Process water must be tested twice daily in triplicate samples using a method approved by the National Shellfish Sanitation Program;

(5)  Treated process water may be used for wet storage tanks, depuration tanks and other processing uses that bring the water into contact with shellfish; and

(6)  Shellfish processed under this subsection may be harvested only from approved or restricted shellfish growing areas that are in the open status as determined by the department.

SUMMARY

This bill establishes conditions under which shellfish depuration certificate holders can be exempt from shellfish emergency rainfall closures.