Be it enacted by the People of the State of Maine as follows:

Sec. 1.  38 MRSA §465, sub-§3, ¶B, as enacted by PL 1985, c. 698, §15, is amended to read:

B.  The dissolved oxygen content of Class B waters shall may not be not less than 7 parts per million or 75% of saturation, whichever is higher, except that for the period from October 1st to May 14th, in order to ensure spawning and egg incubation of indigenous fish species, the 7-day mean dissolved oxygen concentration shall may not be less than 9.5 parts per million and the 1-day minimum dissolved oxygen concentration shall may not be less than 8.0 parts per million in identified fish spawning areas.  Between May 15th and September 30th, the number of Escherichia coli bacteria of human and domestic animal origin in these waters may not exceed a geometric mean of 64 per 100 milliliters or an instantaneous level of 427 236 per 100 milliliters.  In determining human and domestic animal origin, the department shall assess licensed and unlicensed sources using available diagnostic procedures.

Sec. 2.  38 MRSA §465, sub-§4, ¶B, as amended by PL 2003, c. 664, §1, is repealed and the following enacted in its place:

B.  The dissolved oxygen content of Class C water may be not less than 5 parts per million or 60% of saturation, whichever is higher, except that in identified salmonid spawning areas where water quality is sufficient to ensure spawning, egg incubation and survival of early life stages, that water quality sufficient for these purposes must be maintained.  In order to provide additional protection for the growth of indigenous fish, the following standards apply.

(1)  The 30-day average dissolved oxygen criterion of a Class C water is 6.5 parts per million using a temperature of 22 degrees centigrade or the ambient temperature of the water body, whichever is less, if:

(a)  A license or water quality certificate other than a general permit was issued prior to March 16, 2004 for the Class C water and was not based on a 6.5 parts per million 30-day average dissolved oxygen criterion; or

(b)  A discharge or a hydropower project was in existence on March 16, 2005 and required but did not have a license or water quality certificate other than a general permit for the Class C water.

This criterion for the water body applies to licenses and water quality certificates issued on or after March 16, 2004.

(2)  In Class C waters not governed by subparagraph (1), dissolved oxygen may not be less than 6.5 parts per million as a 30-day average based upon a temperature of 24 degrees centigrade or the ambient temperature of the water body, whichever is less.  This criterion for the water body applies to licenses and water quality certificates issued on or after March 16, 2004.

Between May 15th and September 30th, the number of Escherichia coli bacteria of human and domestic animal origin in Class C waters may not exceed a geometric mean of 126 per 100 milliliters or an instantaneous level of 236 per 100 milliliters.  In determining human and domestic animal origin, the department shall assess licensed and unlicensed sources using available diagnostic procedures.  The board shall adopt rules governing the procedure for designation of spawning areas. Those rules must include provision for periodic review of designated spawning areas and consultation with affected persons prior to designation of a stretch of water as a spawning area.

Sec. 3.  38 MRSA §465-B, sub-§2, ¶B, as enacted by PL 1985, c. 698, §15, is amended to read:

B.  The dissolved oxygen content of Class SB waters shall must be not less than 85% of saturation.  Between May 15th and September 30th, the numbers of enterococcus bacteria of human and domestic animal origin in these waters may not exceed a geometric mean of 8 per 100 milliliters or an instantaneous level of 54 per 100 milliliters.  In determining human and domestic animal origin, the department shall assess licensed and unlicensed sources using available diagnostic procedures.  The numbers of total coliform bacteria or other specified indicator organisms in samples representative of the waters in shellfish harvesting areas may not exceed the criteria recommended under the National Shellfish Sanitation Program Manual of Operations, Part I, Sanitation of Shellfish Growing Areas, United State Department of States Food and Drug Administration.

Sec. 4.  38 MRSA §465-B, sub-§3, ¶B, as enacted by PL 1985, c. 698, §15, is amended to read:

B.  The dissolved oxygen content of Class SC waters shall must be not less than 70% of saturation.  Between May 15th

and September 30th, the numbers of enterococcus bacteria of human and domestic animal origin in these waters may not exceed a geometric mean of 14 per 100 milliliters or an instantaneous level of 94 per 100 milliliters.  In determining human and domestic animal origin, the department shall assess licensed and unlicensed sources using available diagnostic procedures.  The numbers of total coliform bacteria or other specified indicator organisms in samples representative of the waters in restricted shellfish harvesting areas may not exceed the criteria recommended under the National Shellfish Sanitation Program Manual of Operations, Part I, Sanitation of Shellfish Growing Areas, United States Food and Drug Administration.

SUMMARY

The bill amends the bacteria standard for class B, C, SB and SC waters and removes a reference to an out-of-date manual, upon the recommendation of the Department of Marine Resources.

The bill makes corrections to and clarifies changes in the dissolved oxygen standard that was enacted in the previous session.